To remove sidecar:

Remove the right saddle bag.

Unhook electrical wire and brake line – both have quick disconnects.

Remove the steering damper completely and set aside (one bolt at front wheel; two bolts at sidecar frame).  Requires 13mm open end wrench and 14mm socket.

Loosen all four bolts that mount the sidecar to the bike but leave them all in place.  Bolts and nuts are ¾”-19mm.  A breaker bar and a good amount of force will probably be required to loosen these nuts.

While one person holds the bike, second person removes the top two bolts.  Then, allow the bike to come to rest to the left on its kickstand.

Support the lower sidecar mounts (I use two inexpensive scissor jacks that I bought from Harbor Freight).

Bolt casters (two) to the front and rear pads on the sidecar frame on the bike side.  I used five inch casters from Lowes, and attached a ½” carriage bolt to them.  You will need to measure to see how long a bolt you need in order to properly support the sidecar, but still be able to insert and remove the casters when while the sidecar is on the jacks.  Measure front and rear; I needed a longer bolt for the rear than the front. NOTE:  I first tried one caster in the middle but the sidecar did not balance well (it was quite nose heavy).

Raise the sidecar wheel using the wheel dolly.  Have one person raise the dolly while the second person makes sure that the car does not slip off the jacks.

Lower the sidecar down onto the casters by lowering and removing the two scissor jacks.

Roll the sidecar away from the bike.  Be sure that you have something available to chock the casters and wheel dolly so it won’t roll away.

To reinstall sidecar:

Roll the sidecar into position as close as possible to the lower mounts on the bike.  NOTE:  When aligning, top sidecar mounts go the rear of the bike mounts; bottom sidecar mounts go to the front of the bike mounts.

Support the two lower mounts on the sidecar with scissor jacks.  Use the jacks to raise the mounts to a good height for inserting the mounting bolts, as well as for removing the casters.

Insert the two lower bolts that attach the sidecar to the bike and run the nuts on loose.  This will usually require one person to move the bike up slightly off the kickstand while the other aligns and inserts the bolts.  A rubber hammer may come in handy here.

Remove the two casters.

Raise the bike to align the top mounts, insert bolts and loose install nuts.

Torque the four mounting bolts to 60 ft-lbs.  The lower mounts will require a long extension for the torque wrench for easy access.

Connect the electrical and brake lines.  Be sure to insert the safety-wired clip to the brake line disconnect.

Reinstall the steering damper.

Reinstall the right saddle bag.